OhioHealth Laboratory Services

QuantiFERON-TB Gold PLUS Collection Instructions

- All collection tubes need to be at room temperature (17⁰-27⁰C) at the time of draw.

- Collect 1 mL of blood into each of the 4 tubes (Gray, Green, Yellow, Purple) provided in the kit.

- If butterfly needle is used, first collect other required tubes, or use a “purge” tube to remove the air and then proceed with collecting the QTB tubes.

- Tubes fill slowly.

- When the tube is upright, blood must be within the small black line on the label.   See figure 1.

- Specimen volumes outside of black mark will be rejected.

- Immediately shake each tube 10 times firmly enough to ensure the entire inner   surface of the tube is coated with blood. See figure 2.

- Do not vigorously shake the tubes. Overly energetic shaking could lead

to gel disruption and cause aberrant results.

- Label all tubes appropriately making sure the correct color label is on the appropriate

colored tube, the QuantiFERON colored band and entire specimen volume is visible.

- Write collection time on ALL tubes.

- Maintain tubes at room temp and transport to RMH Special Testing.

- Tubes must reach RMH Special Testing within 12 hours of collection time

or samples may be rejected.

OhioHealth Laboratory Services

QuantiFERON-TB Gold PLUS Collection Instructions

- All collection tubes need to be at room temperature (17⁰-27⁰C) at the time of draw.

- Collect 1 mL of blood into each of the 4 tubes (Gray, Green, Yellow, Purple) provided in the kit.

- If butterfly needle is used, first collect other required tubes, or use a “purge” tube to remove the air and then proceed with collecting the QTB tubes.

- Tubes fill slowly.

- When the tube is upright, blood must be within the small black line on the label.   See figure 1.

- Specimen volumes outside of black mark will be rejected.

- Immediately shake each tube 10 times firmly enough to ensure the entire inner   surface of the tube is coated with blood. See figure 2.

- Do not vigorously shake the tubes. Overly energetic shaking could lead

to gel disruption and cause aberrant results.

- Label all tubes appropriately making sure the correct color label is on the appropriate

colored tube, the QuantiFERON colored band and entire specimen volume is visible.

- Write collection time on ALL tubes.

- Maintain tubes at room temp and transport to RMH Special Testing.

- Tubes must reach RMH Special Testing within 12 hours of collection time

or samples may be rejected.